Project Proposal: Vision-Reactive Pupper with Motion Tail

Mark Wang | Kurt Lin | Sichen Liang

Motivation

Most quadrupedal robots studies are centered on movement, including gaits, navigation, and path planning; meanwhile, how robots interact with people is often considered less important. However, if you observe most domesticated dogs, the first thing they do upon seeing a human is not solving a maze. A dog will look at the person, walk to them, and begin to wag its tail. This describes the behavior we would like to study with Pupper v3. Social tail wagging is an example of a social behavior we would like to embed in Pupper v3.

Project Description

The primary goal is to make Pupper v3 social. The first step is to make the Pupper approach people and then once the Pupper is within a certain proximity, have the Pupper physically wag a tail. This can be broken down into three parts.

The first part is person detection. Using the on-board camera, the goal is to have the Pupper detect that a person is within camera shot and has an approximate direction and distance. We are currently determining if the HOG+SVM person detector in OpenCV is enough, or if we need to use a more advanced person detector like a lightweight YOLO. HOG is more straightforward to use since it doesn’t require conversion, whereas YOLO would be more accurate in bounding box detection. We will conduct performance metrics on the RPi 5 to determine which is which.

Second is approach behavior. Once a person is detected, the Pupper should orient towards them and walk straight ahead. We intend to accomplish this with a simple visual servoing loop, in which the horizontal displacement of the bounding box drives yaw adjustments to the gait controller, and the bounding box is used as a proxy for distance. Once the bounding box increases (the person is getting closer) the robot slows down and eventually stops at a comfortable distance, which is approximately at the person’s feet. Additionally, the robot is expected to maintain an approximate lock on the person by yawing its body to keep them in the frame whenever the person moves laterally.

Third is the tail. We are developing the tail component through a 3D print of an existing, open-source, cable-driven design from Thingiverse that is driven by a 9g servo we already have. This servo will plug into an empty channel on the Pupper’s servo driver and will require no additional hardware or wiring aside from a simple extension cable. The tail will remain idle in the absence of a person. When the Pupper approaches a person, the tail will begin to wag, with a closer person mapped to a more intense wag.

As a stretch goal, we would like to implement ball detection via color segmentation to allow the Pupper to track and chase a brightly colored ball if we complete the core behavior early.

Performance Metrics

(1) Person detection rate—true positive percentage across 50 frames at 0.5m, 1m, and 2m under indoor lighting. Target: >80% at 1m. (2) Approach success—percentage of trials where the Pupper walks to within 30cm of a stationary person from a 2m starting distance. Target: >70% over 10 trials. (3) Tail response latency—time from person detection to tail wag onset. Target: <500ms.

Timeline

Week 1: Print and mount tail, get person detection running, benchmark OpenCV vs YOLO.

Week 2: Implement approach behavior and person-tracking loop, integrate tail wag trigger.

Week 3: Tune, run metric trials, prepare final presentation.